SCOPE OF WORK FOR CENTRALISED WARE HOUSE IN BEML REGIONAL OFFICE, BILASPUR

GENERAL:

- The works has to be carried-out on Engineering, Procurement & Construction (EPC) i.e. Design & Build concept (on Turn-key basis)

- Total existing Regional office site area:  8.50 Acres approximately (750’ feet length X 500 feet width)

- The available area for development/construction is approximately 1x 750’ X 150’ - 2.63 acres approximately

- The site has to be surveyed for levels,

- Soil testing for arriving SBC at required locations.

- Submission of 02 optional 2D layout plans for the approval of BEML

- Submission of 3D views minimum 2 views for each structure for the finalized layout plan

- Making suitable corrections & submission of final 2D layouts plans and 3D views. Preparation and submission of 3D walk through presentation for the entire project covering roads, hardstanding, common areas, proposed structures etc.

- All the structures have to be designed as per relevant prevailing IS codes.

- Testing of materials has to be carried out as per relevant prevailing IS codes.

- Testing of concrete for compressive strength (for every 30 Cu.M of concrete) for both 7 days & 28 days

- Obtaining licence / approvals / NOC from respective local authorities viz: Municipal / Corporation / Factory Inspectorate / Electric supply company / fire safety etc.  Any statutory fees paid to the respective authorities will be reimbursed by BEML based on production of receipt/ documentary evidence of having paid to the authorities. The contractor has to comply and, if necessary, resubmit applications as required by the local bodies. If required the contractor has to appoint at his own cost, consultants for obtaining local body approval. The documents prepared shall be submitted to BEML for record. All the documents created out of the assignment will become the sole property of the BEML. Applications and receipts submitted to local bodies for statutory approvals shall be submitted to BEML for records.

- All the structures proposed are to be constructed considering floor level a minimum of 600mm above the existing road level.

- The bidders may please note that the proposed site / land/ terrain is at a lower level than the existing road level.

- Preparation of Architectural & Structural Designs of various structures, office accommodations, Entries to each structure, hardstanding all- around structures and buildings, Electrical, toilet blocks, water sump with all connected pipelines for supply of water, sanitary lines connecting to Sewage treatment plant, storm water drains connected to rain water harvesting pond, Electrical works, plumbing works, Fire & safety works, Detailed Designs of Services, external development, roads and pavements, signage’s etc. complete as per requirements. It shall also include modifications, if any, required in the master plan and concept plans to meet the conformance to the local bye laws and the same, shall be done with the approval of BEML LTD

- Preparation of drawing for execution with civil, structural, electrical, plumbing & sanitary, fire, data cabling, telephone services etc.

- Obtaining proof checking of structural designs and drawings from any IITs, NITs, IISC.

- The work shall also include clearance of jungle, cutting of all existing trees as required, services and utilities including removal of debris and foundations of the existing structures including steel structure, concrete platforms, filling up to required levels and site formation etc., complete.

- Supervision of works with required quality assurance.

- All safety norms as per requirement to be ensured by the contractor at site

- Obtaining approval of electrical drawings from Central/ State Electrical Inspectorate, as applicable.

- The scope of work includes dismantling, if any, of the required services and utilities falling in the project area and supporting/shifting & making functional existing services/sewerage and water supply lines etc. The contractor shall properly take care & safe guard all the existing services in the area while execution of work.

- Scope of work contained in the paras mentioned below is only indicative and not exhaustive. In addition, the contractor shall be responsible for executing all items required for completing the building/ development activities in all respects to make it habitable and ready for occupation etc. complete, as per the drawings furnished for execution duly certified.

- Planning, designing and construction/ installation of Under Ground Sump for water supply, Domestic water tank including installing of pumps, stand by pumps, as per approval.

- Storm water drains for proper drainage of rain water to the rain water harvesting system. Any structural design required shall be carried by the tenderer at his own cost. Nothing extra shall be payable in this regard.

- All labour employed shall have ESI and EPF.

- The above scope of work includes cost of all materials, manpower, equipment’s, T&P (Tools & Plants), fixtures, accessories, watch & ward and all other essential elements for completion and maintenance of works as aforesaid whatsoever.

- Illumination at any given point of location should be as per IS standards

- Construction of following buildings/structures are included in the scope of the present tender/work:

| Sl. No. | Description | Type of buildings | No. of Units | L in Ft | W in Ft | Area | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Receiving, holding, packing, dispatch | PEB | 1 | 460 | 150 | 69,000 | Sqft |
| 2 | External Common Toilets | RCC | 2 | 20 | 20 | 800 | Sqft |
| 3 | Security room | RCC | 1 | 20 | 20 | 400 | Sqft |
| 4 | Rain water drainage system | RCC | 1 |  |  | 1 | Job |
| 5 | Roads & hardstanding in open area to withstand 50-ton capacity / load vehicle movement |  | 1 | 50,000 |  | 50,000 | Sqft |
| 6 | New entrance/ security gates | RCC | 1 |  |  | 1 | LS |
| 7 | Sump 25000 litres capacity | RCC | 1 |  |  | 1 | No. |
| 8 | REQUIRED ADDITIONAL ELECTRICAL POWER / LOAD SANCTION, NEW TRANSFORMER: |  |  |  |  | 1 | LS |
| 9 | STP plant 25 KLD along with connecting all Toilets with UGD |  | 1 |  |  | 1 | No. |
| 10 | Dismantling of dilapidated structures and carting away debris |  | 1 |  |  | 1 | LS |
| 11 | EOT 5 Ton along with rails for 50 ft length on either side |  | 1 |  |  | 1 | Nos. |
| 12 | EOT 10 Ton along with rails for 50 ft for 50 ft length on either side |  | 1 |  |  | 1 | Nos. |
| 13 | Providing chain link fencing for a height of 6' & length of 900' approximately with 12 ft wide gates at two locations |  | 1 |  |  | 1 | LS |
| 14 | Covered yard | PEB | 1 | 100 | 80 | 8,000 | Sqft |

SCOPE OF WORK FOR INDIVIDUAL STRUCTURES:

- RECEIVING, HOLDING, PACKING, DISPATCH

- Pre-Engineered Building to be constructed including painting for all steel surfaces

- Size of the structure: Approx. 460 ft X 150 ft

- Gantry level / Crane level: 8 M (approx. 26 ft) near receiving area & dispatch area.

- Roof Truss bottom chord level-9.50 M (approx. 31 Ft).

- Structure has to be designed for carrying minimum 20 Ton capacity EOT cranes Receiving stores area & dispatch store area where EOT cranes are proposed. Rest of the structure is to be designed suitably for store requirement

- Flooring to be designed for bearing a load of 50T vehicle movement at any point of location.

- UPVC Windows in side walls three track with grill & mesh.

- Around masonry wall upto lintel height and balance height with cladding sheets with louvers and mesh.  Internal partition walls separating holding area, packing & dispatch shall be of MS structural support frame covered with galvalume 0.5mm thick on one side upto ridge level

- Turbo ventilators-60 nos to be provided for the entire stores area

- Plastering and painting for masonry walls.

- Enamel paint for all structural steel members of the stores over a coat of primer

- Motorized rolling shutter 7 nos. - each 15 ft wide. Painting for rolling shutters.

- Required Electrical works for the structure, for EOT, lighting, general purposes, including safety aspects etc.

- Roofing & Cladding:  Color coated PUF insulated Galvalume sheets 50 mm thick and poly carbonate sheets 3mm thick for day light in alternate bays/ required intervals etc. for roofing.  Profile galvalume sheets of 0.50mm thick  has to be provided for cladding.

- Office accommodation viz., chamber with false ceiling-01no. with attached toilet, cubicles-04 nos for staff: glazed partitions along with Vitrified tiled flooring including toilet facility. Anti-skid floor tiles, glazed dadoo tiles, required sanitary fitting, plumbing fittings, doors, exhaust fans, ceiling fans, LED light fittings etc.,

- Air-condition for Office/chamber.

- Required fire fighting system with Fire extinguisher to be provided where ever required.

- 4 nos. of HVLS fans to be provided.

- Cubicals of (15 ft X 10ft – 4 Nos), Chamber – 12 ft X 10 ft-1 no.), Toilet – (10ft X 10 ft – 1 no.), Attached toilet (5 ft X 10 ft-1 No.) to be provided

- COMMON EXTERNAL TOILET BLOCK – 2 Nos.

- RCC structure

- Area of toilet block: each approx. 400 Sq.ft

- Internal & external masonry walls with RCC roof slab.

- Plastering and painting for masonry walls, steel and wood/ WPC surfaces

- Required Electrical works for the structure & general purposes, etc

- Glazed tiles for walls & antiskid tiles for flooring in toilet

- HDPE overhead tanks of required capacity, branded pluming, sanitary fittings etc

- Doors – WPC doors with door frames, Ventilators – UPVC, MS grills

- Necessary sanitary fittings to be provided like EWC, IWC, Urinals, Flush tanks, Wash basins, taps, plumbing & sanitary lines etc.

- SECURITY ROOMS – 1 Nos.

- RCC framed structure building

- Area of the building: approx.. 400 Sq.ft with toilet facility

- UPVC Windows in side walls three track with grill & mesh along with roller blinds

- WPC doors with door frames, ventilator grills

- Vitrified tiled flooring

- Internal & external masonry walls with RCC roof slab.

- Glazed tiles for walls & anti skid tiles for flooring in toilet

- Plastering and painting for masonry walls, steel and wood/ WPC surfaces

- Required Electrical works for the structure & general purposes etc.

- HDPE over head tanks of required capacity, branded pluming, sanitary fittings etc.,

- CCTVs installation with required cabling, TV 42” – 45”- 01 no. equipped with necessary accessories

- STORM WATER DRAINS

  - The existing old drainage system if any coming in the way of new proposed drains to be dismantled.

  - Open RCC drains are to be provided with minimum of 6” side walls and along with necessary foundation

  - Deck slabs at required locations viz: entrances of building / structure, cross drains etc. for movement of 50 T capacity wheeled  vehicles.

  - Proper Slope to be maintained for run-off of rain water

- CONCRETE ROADS & HARDSTANDING:

- Provision of new roads, widening of existing roads, hard-standing (open yard) at required locations are to be provided for movement of 50 T capacity wheeled  vehicles

- Approximate area of work to be carried out: approx. 50,000 Sq.ft

- NEW ENTRANCE/SECURITY GATE:

  - Architectural elevation for the main entry to Regional Office premises with ornamental gates & required lighting, granite cladding, required painting works etc

  - Installation of BEML Logo, sign boards etc. with focus lights to be provided.

- SUMP OF CAPCITY 25,000 LITRES – 1 No.

- RCC ground level sump to provided

- Water proofing to be carriedout with necessary ladder

- To be elevated approx.. 1.50 ft above the ground level

- Necessary motor along with pipe lines for lifting water from sump to Over head tank to be provided.

- REQUIRED ADDITIONAL ELECTRICAL POWER / LOAD SANCTION, NEW TRANSFORMER:

- Required electrical additional power sanction has to be obtained.

- Statutory fees will be reimbursed against submission of receipts.

- Steel poles of required height have to be provided for illumination purpose with light fittings.

- Under ground cabling work to be carried out with control switches at required locations.

- SEWAGE TREATMENT PLANT OF 25 KLD CAPACITY:

- RCC structure to be provided for STP plant for required operations at suitable location.

- Plant & machinery to be provided

- UGD lines along with Manholes have to be provided from all sanitary outlets of buildings to STP plant

- Electrical connections along with cabling, lighting etc., to be provided

- Outlet drains have to be provided.

- DISMANTLING OF DILAPIDATED STRUCTURES AND OLD STRUCTURES

- The old, dilapidated structures existing in the proposed area are to be  demolished

- The debris has to be shifted and carted away outside BEML premises

- EOT- 5 TON OF REPUTED MANUFACTURER – 1 No.

- Supplying, installation, testing and commissioning of 5 Ton with rails (for 50 feet length both ways) in Receiving stores area

- Required electrical connection to EOT and trail run

- Gantry level / Crane level: 8 M (approx. 26 ft) only for 50 feet crane movement length.

- EOT- 10 TON OF REPUTED MANUFACTURER – 1 No.

- Supplying, installation, testing and commissioning of 10 Ton with rails (for 50 feet length both ways) in Dispatch stores area

- Required electrical connection to EOT and trail run

- Gantry level / Crane level: 8 M (approx. 26 ft) only for 50 feet crane movement length.

- CHAIN LINK FENCE WITH ENTRY & EXIT GATES -APPROX.900 Ft. LENGTH

- GI Chain link fence of 50mm X 50mm X 2mm gauge with 35 GSM COATING with alround MS famed structure supports, vertical MS structural steel members for holding the fence. Approx. 900 Ft Length and 6Ft Height

- MS Gates -02 nos 12 Ft wide each for entry and exit.

- Enamel paint for all MS members over a coat of primer

- Necessary civil works for fence-foundation, PCC, RCC & masonary work

- Plastering & painting for masonary & concrete members

- COVERED YARD

- Pre-Engineered Building to be constructed

- Size of the structure: Approx. 100 ft 80 ft

- Roof Truss bottom chord level-6.00 M (approx. 20 Ft).

- Flooring to be designed for bearing a load of 50T vehicle movement at any point of location.

- Alround masonry wall upto 4 feet masonry wall  Necessary entrance to be provided.

- Plastering and painting for masonry walls.

- Enamel paint for all structural steel members of the stores over a coat of primer

- Required Electrical works for the structure, lighting, general purposes, including safety aspects etc.

- Roofing & Cladding:  Color coated PUF insulated galvalume sheets 50mm thick and profile galvalume sheets of 0.50mm thick for cladding upto 1.00M below the bottom chord level.